STBLink 2.03 User Manual

Last updated 2026.07.28

# 0. Revision History

| Date | Versions | Author | Descriptions |
| --- | --- | --- | --- |
| 2006/03/28 | V1.0 |  | Initial version |
| 2008/03/19 | V2.0 |  | Edit version |
| 2008/04/04 | V2.1 |  | Addition Move Channel and softkey.kdb. version. |

[ STBLink 02.xx ]

- Preparation of equipment

- Set-top Box, RS232 Cross Cable

2.  Basic download Process

- Connect RS232 cable (Cross cable) PC to STB.

- Change to Stand-by status by power button(key).

- Set COM(1~4) port, connection.

- Click the Icon or Menu that you want to download.

3.  Help

[ table of contents ]

1.   COM 1~4  (PC Com-port Setting)

2.   Connect   (Connection between STB and STBLink02.xx)

3.1  SW Down  ( Software download at Normal booting Status )

3.2  SW Down  ( Software download at Boot-mode Status )

4.1  TV.CH  ( TV, Radio CH and other information upload)

4.2  TV.CH  ( TV, Radio CH and other information download)

5.   CAS  (softcam.key or softcam.kdb download)

6.   Image  (Background Image download)

7.   Boot Loader  (Boot(Code) Loader download)

8.   New UDB Data  (New UDB Data)

9.   Load UDB Data  (Load user UDB Data)

10.   Save UDB Data  (Save user UDB Data)

11.   Edit Satellite  (Edit for choose satellite)

12.   Add transponder  (Additional new transponder)

13.   Delete transponder  (Delete transponder the choose transponder )

14.   Edit transponder  (Edit for choose transponder)

15.   Edit favorite Group  (Edit for choose favorite group name)

16.   Delete favorite channel  (Delete channel the choose favorite channel)

17.   Edit favorite channel  (Edit for choose channel)

18.   Add channel  (Additional new channel)

19.   Delete channel  (Delete channel the choose channel)

20.   Edit channel  (Edit for choose channel)

21.   Add Fav channel  (Add the choose channel to favorite channel)

22.   Move channel  (Move for choose channels)

23.   Link Key Editor  (Run Key Editor)

* reference : [ pic2mpg image converter ]

1.  COM 1~4  (PC Com-port Setting)

a. Click          icon at STB standby status. ( see below )

b. if it has problem, check as below.

2.  Connect   (Connection between STB and STBLink02.xx)

a. Click         icon at STB standby status. ( see below )

Connection OK.

b. if it has problem, check as below.

3.1  SW Down  ( Software download at Normal booting Status )

a. Click          icon at STB standby status. ( see below )

Press  [Normal Status (S/W)]  menu button

b if you want to continue, Press “OK”. Otherwise Press “Cancel”.

c. choose the file(*.BIN) that you want to download.

d. if Software version is correct, it will continuously processed.

- Otherwise , warning message will be displayed as below.

e. After transferred main image, please waiting for writing image to Flash.

f. then reboot.

3.2  SW Down  ( Software download at Boot-mode Status )

- As a first step, power off->on while press ch- button on front-panel.

On front-panel, it will be displayed as below. (x = character)

00xx -> 001x -> 1 -> 001x -> 1 … (loop continuously)

b. Click          icon. ( see below )

Press  [Boot-mode Status (S/W)]  menu button.

c. if you want to continue, Press “OK”.  Otherwise Press “Cancel”.

d. choose the file(*.BIN) which you want to download.

e. if Software version is same, it will be continuously process.

- Otherwise , It will be displayed warning message as below.

f. After transferred main image, please waiting for writing image to Flash.

g. then reboot.

4.1  TV.CH  ( TV, Radio CH and other information upload)

a. Click         icon at STB standby status. ( see below )

b. if you want to continue, Press “OK”.  Otherwise Press “Cancel”.

c. connect and upload User data(TV, Radio, satellite data.. etc).

d. if all data is transferred, please save User-data file(*.UDB). then reboot.

4.2  TV.CH  ( TV, Radio CH and other information download)

a. Click         icon at STB standby status. ( see below )

b. if you want to continue, Press “OK”.  Otherwise Press “Cancel”.

c. choose the file(*.UDB) which you want to download.

d. User data(TV, Radio, satellite data.. etc) Downloading.

e. After transferred user data, please waiting for writing image to Flash.

f. then reboot.

5.  CAS  (softcam.key download)

a. Click         icon at STB standby status. ( see below )

b. if you want to continue, Press “OK”.  Otherwise Press “Cancel”.

c. Choose the file(softcam.key or softcam.kdb) which you want to download.

d. Softcam.key downloading.

e. After transferred userdata, please waiting for writing image to Flash.

f. then reboot.

6.  Image  (Background Image download)

a. convert [pic, jpg, bmp, tif] to [mpg] by using pic2mpg.exe. (Image convert program)

* reference pic2mpg image converter process.

b. Click         icon at STB standby status. ( see below )

if you want to continue, Press “OK”.  Otherwise Press “Cancel”.

c. Choose the file(*.mpg) which you want to download.

d. Background Image downloading.

e. After transferred userdata, please waiting for writing image to Flash.

f. then reboot.

7.  Boot Loader  (Boot(Code) Loader download)

a. Click                          at STB standby Status. ( see below )

b. if you want to continue, Press “OK”.  Otherwise Press “Cancel”.

c. choose the file(*.BIN) that if you want to download.

d. downloading BootLoader.

e. after transferred userdata, please waiting for writing image to Flash.  then reboot.

8.   New UDB Data  (New UDB Data)

a. Click   at  new UDB Data icon. 
or click  at User Database menu.

9.   Load UDB Data  (Load user UDB Data)

- Click  at Load UDB Data icon. 
or click  at User Database menu.

- Choose the file(*.UDB) which you want to edit. ( see below )

- if it has problem, check as below.

10.   Save UDB Data  (Save user UDB Data)

- Click   at Save UDB Data icon. 
or click  at User Database menu.

- Save the file(*.UDB) which you want to save. ( see below )

- if it has problem, don’t save popup windows.

11.   Edit Satellite  (Edit for choose satellite)

- First, select satellite which you want choose.

- And then Click  at Satellite menu. 
or click  mouse right button at Satellite list. And click Edit satellite menu.
or double click at Satellites list of line.
or press enter key at Satellites list of line.

- And then popup Satellite Information edit window. ( see below )

- Edit satellite items.

- And the if you want to continue, Press “OK”.  Otherwise Press “Cancel”.

12.   Add transponder  (Additional new transponder)

- First select you want to the satellite.

- And select you want to the transponder list of line.

- And then Click  at Transponder menu. 
or click  mouse right button at Transponder list. And click Add transponder menu.

- And then popup Transponder Information at edit window. ( see below )

- Edit transponder items.

- And the if you want to continue, Press “OK”.  Otherwise Press “Cancel”.

13.   Delete transponder  (Delete transponder the choose transponder )

- First select that you want to the satellite list of line.

- And select that you want to the transponder list of line.

- And then Click  at Transponder menu. 
or click   mouse button at Transponder list. And click Delete transponder menu.

- And then popup Confirm Information window. ( see below )

- And the if you want to continue, Press “OK”.  Otherwise Press “Cancel”.

14.   Edit transponder  (Edit for choose transponder)

- First select that you want to the satellite list of line.

- And select that you want to the transponder list of line.

- And then Click  at Transponder menu. 
or click  mouse right button at Transponder list. And click Edit transponder menu.
or double click at Transponder list of line.
or press enter key at Transponder list of line.

- And then popup Transponder Information edit window. ( see below )

- Edit transponder items.

- And the if you want to continue, Press “OK”.  Otherwise Press “Cancel”.

15.   Edit favorite Group  (Edit for choose favorite group name)

- First select that you want to the Favorite Group list of line.

- And Click  at Fav Group menu. 
or click   mouse right button at Favorite Group list. And click Edit favorite group menu.
or double click at Favorite Group list of line.
or press enter key at Favorite Group list of line.

- And then popup Favorite Group Information edit window. ( see below )

- Edit Favorite Group name items.

- And the if you want to continue, Press “OK”.  Otherwise Press “Cancel”.

16.   Delete favorite channel  (Delete channel the choose favorite channel)

- First select that you want to the Favorite Group.

- And Click  at Favorite Channel menu. 
or click  mouse right button at Favorite channel list. And Click Delete favorite channel menu.
or press delete key at Favorite Channel list of line.
or choose the favorite channels and then drop and drop to All Channel list. ( see below )

- And then popup Confirm Information window. ( see below )

- And the if you want to continue, Press “OK”.  Otherwise Press “Cancel”.

17.   Edit favorite channel  (Edit for choose channel)

- First select you want to the Favorite Channel list of line.

- And Click  at Favorite Channel menu. 
or click  mouse right button at Favorite channel list. And click Edit favorite channel menu.
or double click at Favorite Channel list of line.
or press enter key at Favorite Channel list of line.

- And then popup Channel Information edit window. ( see below )

- Edit Channel items.

- And the if you want to continue, Press “OK”.  Otherwise Press “Cancel”.

18.   Add channel  (Additional new channel)

- Click  at Channel menu. 
or click  mouse right button at All channel list. And click Add channel menu.

- And then popup Channel Information edit window. ( see below )

- Edit Channel items.

- And the if you want to continue, Press “OK”.  Otherwise Press “Cancel”.

19.   Delete channel  (Delete channel the choose channel)

- First Select that you want to the Channel list of line.

- Click  at Transponder menu. 
or click  mouse right button at All channel list. And click Delete channel menu.
or press delete key at Channel list of line.

- And then popup Confirm Information window. ( see below )

- And the if you want to continue, Press “OK”.  Otherwise Press “Cancel”.

20.   Edit channel  (Edit for choose channel)

- First select that you want to the Channel list of line.

- And Click  at Channel menu. 
or click  mouse right button at All channel list. And click Edit channel menu.

- And then popup Channel Information edit window. ( see below )

- Edit Channel items.

- And the if you want to continue, Press “OK”.  Otherwise Press “Cancel”.

21.   Add Fav channel  (Add the choose channel to favorite channel)

- First select you want to the All Channel list of line.

- Click  at Channel menu. 
or click  mouse right button at All channel list. And click Add Fav channel menu.
or choose the channels and then drop and drop to Favorite Channel list. ( see below )

- if it has already exist favorite channel, check as below.

22.   Move channel  (Move for choose channels)

- First select you want to the All Channel list of choose the channels.

- And then drop and drop to All Channel list. ( see below )

23.   Link Key Editor  (Run Key Editor)

- Click   icon.  Then run KeyEditor( see below ).

- Edit Key item.

[ pic2mpg image converter ]

- execute Pic2Mpg.exe program, you can see as below.

Click file-Open button for Image Convert.

2.  Choose the file(jpg, bmp, gif, tif) which you want to convert.

3.  if you choose the file, Press [Image Convert] button.

4. then, it will be display as below.

Image Convert -> Converting… wait -> convert completed.

In current directory, it will be generated one file with *.mpg extension.

Ex ) ColorNameChart.mpg